Real-Time Traffic Monitoring and Violation Detection Using YOLO and the UA-DETRAC Dataset

Priyanka Gupta 
School of Computer Science and Engineering

Lovely Professional University

Phagwara, India
priyankag8048@gmail.com

Ajay Kumar Reddy

School of Computer Science And Engineering
Lovely Professional Universisty
Phagwara,India
ajaykumarreddy866@gmail.com

Abstract—With rapid urbanization and growth of vehicular networks, traffic management has turned into a far more complicated problem for contemporary cities. Traffic offences like speeding and jumping the signal are prime causes of road accidents, traffic jams, and inefficiencies in law enforcement. This study proposes an AI-based, real-time traffic violation monitoring and detection system that leverages the capabilities of deep learning and computer vision for automatically detecting such violations through video surveillance. The suggested system takes advantage of the UA-DETRAC dataset, which has annotated traffic videos, to train and evaluate the YOLOv8 object detection model. YOLOv8 is utilized for its trade-off between speed and accuracy in real-time scenarios. Our approach entails the detection of vehicles frame by frame, computation of their speed based on pixel displacement with respect to time, and observation of their location with respect to an assumed stop line to identify red signal violations. A virtual stop line is placed within the video stream to provide an imitation of traffic lights, and a pixel-based speed threshold is used to detect over-speeding vehicles. Violations are detected and recorded in real time and give both visual and text-based alerts. The system proves efficient in correctly detecting and reporting various traffic violations, thus proving its suitability for implementation in smart city infrastructure. This work adds to the body of intelligent transportation systems by introducing a low-cost and scalable solution for traffic violation detection. The results highlight the feasibility of integrating sophisticated object detection with basic tracking heuristics to create robust traffic law enforcement tools. Real-world calibration for speed estimation, vehicle re-identification, and integration with license plate recognition for automated ticketing are some of the possible future improvements.

Keywords—Traffic Violation Detection, YOLOv8, UA-DETRAC Dataset, Real-Time Surveillance, Signal Jumping, Speed Estimation, Computer Vision, Smart City, Deep Learning, Object Detection

# Introduction

Over the last few years, the rapid growth of urbanization has grown the number of vehicles on roads exponentially, thereby doubling the issues related to traffic management and law enforcement. Traffic offences like over-speeding and jumping red lights have become the order of the day, which has not only led to road accidents and traffic jams but also eroded public confidence in transport systems. These challenges are further compounded by the inherent constraints of conventional traffic surveillance methods, which tend to rely on human observation, fixed cameras, or individual sensor deployments—each with limited reach and vulnerability to mistakes or inefficiencies. With cities pushing to become "smart cities," there is increased need for scalable and automated solutions that can accommodate real-time decision-making and monitoring. Artificial Intelligence (AI) in the form of deep learning and computer vision has been the game-changer in this arena. Sophisticated object detection algorithms can now reliably detect and track vehicles over frames in a video stream, paving the way for behaviour analysis and violation detection.

The YOLO (You Only Look Once) family of models has proven itself to be one of the best methods for object detection based on its speed and accuracy, particularly in real-time settings. The most recent and optimized version, YOLOv8, provides additional advancements in model design, training approaches, and detection accuracy. Based on YOLOv8, this study proposes a new AI-based system for real-time traffic violation detection using publicly available video data from the UA-DETRAC dataset. The UA-DETRAC dataset is selected because it is rich in annotated traffic scenarios, such as multiple types of vehicles, conditions of the environment, and city traffic patterns. Our system identifies vehicles frame by frame, traces their movement, estimates speed from pixel displacement, and tracks their interaction with a synthetic stop line to check if any traffic laws are being violated. Different from many previous research that considers either a single category of violation (often speeding or lane departure) separately, in this system the speeding and the red-light signal jumping detection are both combined together into a streamlined and effective pipeline. The proposed system is architected to process solely video inputs—without having any external sensor or hardware dependency—and hence offers extreme deploy-ability and scalability into multiple traffic observation settings. With this work, we seek to advance the area of intelligent traffic systems through the demonstration of the deploy-ability of AI architectures such as YOLOv8 for real-world, real-time enforcement applications. The system not only enhances the effectiveness of traffic surveillance but also paves the way for future developments such as license plate reading, multi-camera coordination, and calibrated speed conversion.

This work is a major contribution to intelligent traffic monitoring through the provision of an AI-based system that can identify multiple traffic offenses based on real-time video analysis. The main contributions of the paper are as follows:

Unified Violation Detection Framework:

Dual Violations at Once: This work is novel in developing a combined system that can detect two kinds of traffic violations at once, namely speeding and jumping the signal. Both the dual functionality aspect is not addressed in current systems, which usually handle the detection of speed or signal-related violations independently. Both of these are combined into a single pipeline here, facilitating multiple types of violations being monitored in real time in an efficient manner without having to use distinct systems for each violation.

System Efficiency: With these two functions merged into a single model, the system minimizes the necessity for several processing units and increases efficiency in general in real-time traffic observation, allowing it to be scalable for implementation in urban settings.

Integration of YOLOv8 for High-Precision Vehicle Detection

Accuracy and Speed Balance: The YOLOv8 model, being fast and accurate, forms the core of the system. YOLO (You Only Look Once) is known for its speed in detecting objects with great accuracy. YOLOv8, as a better version, provides better performance in object detection accuracy as well as in speed needed for real-time applications. It is used to process complicated traffic scenes where cars with varying sizes and speeds must be followed between frames, so the system can process videos at a fast rate while keeping high detection efficiency.

Multiple Vehicle Detection: The system is able to detect and track numerous types of vehicles such as cars, buses, trucks, and motorcycles, making it suitable for various urban road conditions. This versatility is essential in real-world deployments where traffic includes different categories of vehicles.

Virtual Stop Line and Signal Phase Simulation

Simulation of Traffic Signal Behaviour: The system adds a virtual stop line in the video stream to simulate traffic light behaviour, which is essential for identifying red light violations. As direct signal information is not available in the dataset, this simulated method imitates vehicle behaviour based on signal changes, giving a safe way to identify if a vehicle passes the stop line during a red light.

Robustness Without Access to Traffic Signal Data: Differing from conventional systems that are based on actual traffic signal data, this virtual stop line method is robust in settings where signal data may not be easily accessible, e.g., in publicly available datasets such as UA-DETRAC.

Pixel-Based Speed Estimation Without Physical Calibration

Simplified Speed Estimation: Another of the major innovations of this system is the capability to estimate vehicle speed directly from pixel displacement in video frames. In contrast to systems that need physical calibration (such as radar or GPS sensors), this approach applies motion analysis and simple geometry to estimate speed in pixel units, making it a desirable solution in situations where resources for calibration or additional sensors are not available.

Scalable and Adaptable: The method is readily transferrable to various datasets or video streams so that the system can be utilized in multiple geographic regions and traffic conditions without large-scale recalibration.

Validation on the UA-DETRAC Dataset

Real-World-like Testing: The system was tested on the UA-DETRAC dataset, which is commonly used for its realistic representation of real-world traffic scenarios, including different types of vehicles, traffic conditions, and environmental conditions (e.g., varying weather conditions and lighting). This is a good validation of the system's performance under a wide range of conditions.

Realistic Traffic Modelling: The UA-DETRAC dataset is appropriate for this use case since it contains both labelled vehicles and realistic traffic scenes, which enables the system to learn strong models for vehicle detection and tracking in difficult situations.

Real-Time Alerting and Logging Mechanism

Instant Violation Logging and Alerting: The system includes a real-time alerting functionality that offers instantaneous feedback when a violation is found. The system marks violations like speeding or jumping a red light by logging the time and information of the violation, which can subsequently be utilized for enforcement or further analysis. This capability in real-time replicates the requirements of a functional smart city infrastructure, where traffic violations must be sensed, logged, and responded to in a timely manner.

Scalability in Monitoring: The real-time logging capability can be scaled to monitor larger regions with little delay, offering timely alerts that could be incorporated into traffic control systems or law enforcement systems.

# KEY FEATURES

## One of the key strengths of the proposed Edge AI-powered traffic violation detection system is its ease of deployment and use. Unlike traditional surveillance systems that require complex infrastructure, dedicated servers, and continuous internet connectivity, this system is designed to be plug-and-play and autonomous.

## Key Features That Enhance Usability

1. Dual-Function Violation Detection

The system is a blend of two large categories of traffic violations, speeding and jumping over signals, that it detects exclusively. This it achieves through observation of both speed and location within traffic signals. The dual-detection ability places the system at a very high level of usability in actual traffic control. By employing a single camera stream, the system is able to track vehicle speeds and traffic signal compliance (e.g., whether a vehicle crosses the stop line while a red light is present) simultaneously.

Speeding Detection: Vehicles are tracked for any movement above a predefined speed threshold (in pixels/frame).

Signal Jumping: The system verifies whether vehicles cross a virtual stop line while a red light is present, mimicking actual traffic light patterns.

2. YOLOv8-Based Object Detection

The model takes advantage of YOLOv8 (You Only Look Once), which is one of the most efficient and widely used object detection models due to its high speed and real-time performance. YOLOv8 has a deep learning architecture, making it able to identify many forms of vehicles, including cars, trucks, and buses, with high accuracy.

Why YOLOv8?: YOLOv8 is extremely efficient at detecting objects rapidly without compromising accuracy. This makes it perfect for real-time video monitoring, where frames must be processed in real-time to detect breaches as they occur.

3. Virtual Stop Line for Signal Jump Detection

To identify signal jumping (running red lights), the system employs a virtual stop line in the video stream. The stop line serves as a reference point to mimic traffic light phases. If a vehicle passes through this stop line during a red signal, the system identifies it as a violation.

Customizable Stop Line: The digital stop line can be set to correspond to the physical stop line position in the real-world setup, hence making it compatible with various traffic configurations.

Simulated Traffic Signal: By software development of red and green signal phases, the system is able to analyze vehicle conduct dynamically as they approach the virtual stop line.

4. Pixel-Based Speed Estimation

Speed estimation is significant in identifying overspeeding cars. Rather than needing to incorporate specialized speed sensors, the system estimates vehicle speed from pixel movement between frames.

How It Works: By observing the movement of cars between frames, the system computes the pixel distance travelled and divides it by the frame-to-frame time. The outcome is an estimate of the car's speed in pixels per frame.

Advantages: Since it does not need external sensors or calibration, this pixel-based method is inexpensive and simple to deploy as a means for speed detection. Real-world calibration may be included later in order to improve accuracy, though.

5. Real-Time Violation Logging and Alerts

Real-time monitoring and feedback about traffic infractions are given by the system. Upon detection of a violation (like speeding or jumping a signal), it is logged along with the date and time and vehicle details.

Timestamped Logs: Each violation is timestamped so that it can be easily accessed, allowing tracking of when the violation took place in the video stream.

Visual Alerts: Violations are visually indicated in the video stream, either by bounding boxes around the offending vehicle or by other types of visual annotation. This provides instantaneous feedback to monitoring staff or automated systems.

6. Cost-Effective and Sensor-Free

This system does not need extra hardware such as speed radar guns or ground sensors. It is based entirely on video data to identify infractions. This is more cost-friendly for large-scale implementation in smart city infrastructures because it avoids expensive external sensors.

No Special Sensors: In contrast with conventional systems that use inductive loops or radar, this system employs cameras as the main source of information.

Minimal Installation: The system has minimal installation requirements of video cameras and a processing computer, saving on installation and maintenance expenses.

7. Flexible Traffic Signal Simulation

The system provides for the simulation of traffic light phases so that dynamic stop line evaluation according to red, yellow, or green signals can be performed. This can be customized to meet the unique demands of various intersections.

Traffic Signal Phases: Users may specify the phases (red, yellow, green) and the length of each signal so that the system can emulate the actual traffic light operation.

Stop Line Position Flexibility: The position of the virtual stop line may be changed in accordance with the actual placement of traffic signals or stop lines, making it flexible for use in different environments.

8. Lightweight and Efficient

While having tremendous capability, the system is implemented to operate efficiently with low computational overhead. This is necessary for real-time usage, where delays in processing can cause violations to go undetected.

Real-Time Processing: The system can process video streams in real time, which means that it can identify violations as they occur without substantial delay.

Low Latency: Through the effective utilization of the YOLOv8 model along with pixel-based speed estimation, the system ensures low latency, such that there is quick detection and alerting of traffic offenses.

9. Modular and Scalable Design

The design of the system takes scalability into consideration. It is modular in nature, such that new features and functionalities can be easily added to the current pipeline.

Easy Integration: New features such as license plate recognition, multi-camera support, or other traffic analysis tools can be integrated easily.

Multi-Camera Support: The software can be easily extended to handle multiple camera inputs for larger intersections or road networks, providing more extensive traffic monitoring.

10. Scalable Across Different Environments

The system can adapt to all types of traffic situations, from complex traffic patterns at urban intersections to sparse traffic conditions on rural roads. Its versatility allows it to be applied to a wide range of purposes in high-density as well as low-density traffic locations.

Urban and Rural Deployment: From the crowded city centre to the desolate suburban street, the system can be tuned to accommodate different traffic conditions and surveillance requirements.

Flexible to Different Camera Configurations: The system is capable of operating efficiently in single-camera configurations for smaller intersections as well as multi-camera configurations for larger networks, thus being very scalable.

# LITERATURE REVIEW

Automatic traffic monitoring and offense detection have emerged as increasingly essential measures in view of the augmented volume of cars on roads and the complexity inherent in manual checks. In this section, an overview is taken of existing work on four prominent aspects that fall under this investigation: vehicle detection, multi-target tracking, estimation of speed, and holistic traffic offense detection systems. A further aspect concerns the benchmark data set.

- Deep Learning Based Vehicle Detection

Previous vehicle detection methods used hand-engineered features and classifiers like Haar cascades, HOG + SVM, and background subtraction, which were mostly not robust against changing illumination, occlusions, and cluttered scenes. The emergence of deep learning, particularly convolutional neural networks (CNNs), has greatly enhanced detection robustness and accuracy. Among the cutting-edge techniques, the YOLO (You Only Look Once) system introduced by Redmon et al. [1] provided real-time performance by predicting class probabilities and bounding boxes using a single neural network. Later variants, such as YOLOv2, YOLOv3, and YOLOv4, enhanced detection speed and accuracy. YOLOv5, introduced by Ultralytics, provides optimized training performance and real-time inference, which makes it ideal for embedded systems. Its light-weight versions, like YOLOv5s, are especially efficient for traffic monitoring tasks because of their speed-accuracy balance.

- Multi-Object Tracking (MOT)

To track vehicle motion over time, object tracking needs to be accurate. Conventional tracking algorithms like Kalman Filters, Mean Shift, and Optical Flow are not robust in high-density or occluded scenes. SORT (Simple Online and Realtime Tracking) was proposed to solve some of these limitations by merging a Kalman Filter with the Hungarian algorithm for data association [2].Yet, SORT depends only on spatial data, resulting in constant identity swaps. To improve performance, DeepSORT [3] built upon this model by incorporating appearance descriptors obtained through a CNN-based Re-Identification (ReID) model. This allowed for improved identity retention under occlusions and cluttered backgrounds. DeepSORT is still one of the top-performing tracking algorithms in applications where real-time performance and robust identity retention are essential.

- Speed Estimation in Vision-Based Systems

Speed estimation is an important aspect of traffic violation detection, especially to detect overspeeding vehicles. Conventional methods use RADAR or LIDAR, but these are costly and only offer restricted coverage. Vision-based solutions offer both accuracy and affordability, and the ability to scale.

One popular technique tracks the vehicle's centroid from frame to frame and estimates displacement over time from the frame rate and an approximated pixel-to-meter ratio [4]. This dimension can be computed through scene calibration by hand or through use of known measurements within the video (e.g., lane width or car length). Errors come about with a presumed constant camera view or scene layout. Some of the latest publications suggest utilizing homography transformations, vanishing points, or 3D reconstruction of scenes to do dynamic calibration but such at the expense of requiring computational capacities above real-time requirements.

- Integrated Violation Detection Systems

In spite of recent progress in detection, tracking, and speed estimation, few systems combine these elements into an end-to-end real-time violation detection pipeline. Current research tends to consider one module in isolation, which hampers its effective deployment. For example, Rastegar et al.  presented a deep neural network- and image processing-based system to identify red-light offenses, but it did not incorporate dynamic speed estimation or tracking of vehicles. In the same way, Patil et al. [6] presented an object detection-based traffic counting system but not the speed or identity retention across frames. The difficulty in the construction of end-to-end systems is obtaining real-time performance coupled with accuracy on real-world settings like occlusion, varying light, and dense traffic. Further, most systems are not scalable because they depend on manual calibration or hardware-level optimizations.

- Benchmark Datasets and Evaluation Protocols

Accurate datasets are required for training and testing smart traffic monitoring systems. Zhang et al. [7] provided one of the most extensive benchmarks, the UA-DETRAC dataset, for vehicle detection and multi-target tracking. It contains more than 100 video clips under various conditions (cloudy, sunny, rainy, and nighttime), and it is annotated with 8250 vehicles and more than 1.2 million frames. UA-DETRAC facilitates standardized testing protocols, thus being particularly well-suited to compare the performance of object trackers and detectors on difficult real-world cases. The presence of occlusions, different vehicle classes (car, bus, van, others), and viewpoints in the dataset makes it particularly useful to create robust and generalizable traffic surveillance systems.

As yet, although the methods of detecting and tracking vehicles have come a long way, integrating them into real-time violation detection of traffic—particularly speeding—continues to be a developing field. The current alternatives have either computational demands that are excessively high or cannot work within unconstrained situations. This paper attempts to close this gap by developing a real-time, modular, and scalable system that integrates YOLOv5 for vehicle detection, DeepSORT for identity-preserving tracking, and a centroid-based speed estimation algorithm. The system is trained and evaluated on the UA-DETRAC dataset, showing that real traffic enforcement solutions are possible with lightweight, open-source tools.

# METHODOLOGY AND IMPLEMENTATION

The goal of the new system is to build a stable, real-time, and scalable system architecture for automatic traffic violation detection with emphasis on speed limit violations and red-light jumping tendencies. The system uses an ensemble of deep learning-based object detection, light-weight tracking, motion analysis, and rule-based violation reasoning, all integrated within an efficient and modular video analytics pipeline.

3.1 System Architecture and Processing Pipeline

The entire system is organized as a multi-stage video analytics pipeline, with each stage designated for a particular computational task. The structure has a unidirectional data flow from video ingestion to end visualization and violation reporting. The system processes sequentially input surveillance video, extracting key spatio-temporal features from traffic video in real time. The pipeline can be described in six logical stages:

- Video Input & Preprocessing

- Vehicle Detection using YOLOv8

- Object Tracking with Pseudo-ID Assignment

- Speed Estimation Using Frame-to-Frame Displacement

- Red-Light Violation Detection via Spatial Rule Checking

- Annotation, Violation Logging, and Real-Time Visualization

Each module in this pipeline is developed separately so that it may be optimized or replaced independently of the others with ease, without affecting the functioning of the system.

3.2 Dataset Selection and Frame Acquisition

The UA-DETRAC dataset is chosen to be the core source of surveillance video data. The dataset contains high-resolution (960×540) annotated traffic videos captured at 25 frames per second from fixed roadside and intersection-mounted cameras. The dataset contains more than 1,200 minutes and 1.2 million manually annotated bounding boxes, spanning dense and sparse traffic conditions under four different weather conditions: sunny, cloudy, rainy, and night.

For real-time processing, the video is ingested through OpenCV's cv2.VideoCapture() API, reading frames in a loop. Every frame is resized and normalized prior to being input to the object detection model. The frame timestamp and index are saved for subsequent temporal computation.

  - Vehicle Detection Using YOLOv8

Vehicle detection is performed with YOLOv8n, the nano model of Ultralytics' YOLOv8 series. YOLOv8 is an anchor-free, next-generation object detection framework with a decoupled head that predicts bounding box coordinates, class probabilities, and confidence independently. The benefits are: Real-time inference on common CPUs and edge GPUs. Smaller model size (~5MB for YOLOv8n), ideal for embedded applications. Better localization accuracy thanks to improved bounding box regression and non-max suppression (NMS).

The model is pre-trained on COCO and fine-tuned over annotated frames in UA-DETRAC to enhance domain-specific generalization. Inference results for each frame comprise the following:

Bounding Box Coordinates:(xmin​,ymin​,xmax​,ymax​)

Class Label: car, bus, truck

Confidence Score: range from 0 to 1

These detections are saved as structured dictionaries per frame for tracking and speed analysis.

3.4 Vehicle Tracking through Pseudo-ID Assignment

For preserving identity between frames, a light-weight pseudo-tracking algorithm is used. Rather than using computationally costly appearance-based trackers such as DeepSORT, this uses centroid-based association between frames:

The center point of each detected bounding box is calculated as:

A pseudo-ID is determined by matching centroids in frame t with that in frame t−1, using Euclidean distance and minimum thresholding. If the centroid of a vehicle is within a distance limit (e.g., <50 px) of a previously tracked vehicle of the same class, the vehicle is considered as tracked.

This approach is efficient in computation but has the drawback of ID switching, particularly when there are occlusions. Nevertheless, it is effective for short-term tracking needed in speed and red-light violation detection.

Future Scope: Integrating robust appearance descriptors and Kalman filtering can improve identity persistence and trajectory smoothing.

3.5 Speed Estimation Methodology

Speed estimation is a core feature and is calculated based on frame-to-frame displacement of tracked vehicle centroids. The mathematical formulation is:

Where (x1,y1) and (x2,y2)are centroids in frames t1​ and t2​, and Δt=1/FPS​.. The result, in pixels per second, is converted to meters per second using an estimated pixel-to-meter ratio α, derived from the known average vehicle length (approx. 4.5 m for a sedan) observed in the frame:

Cars that have a speed over a certain threshold (e.g., 15 m/s or 54 km/h) are indicated as having a speed violation. The ratio α is calibrated manually and can be dynamically estimated with vanishing point-based homography or camera calibration methods in future versions.

3.6 Red-Light Violation Detection Logic

Red-light violation detection is carried out by means of a virtual stop line and a global traffic light status flag. A stop line is represented as a horizontal line with a fixed Y-coordinate, ystop, which is manually specified according to the road layout in the frame. The system emulates a red light by putting the status of the traffic light to RED during specified frame intervals. A violation is defined when the bottom edge of a car's bounding box moves past this stop line while the light is RED:

This basic rule-based detection works well in controlled scenarios and may be supplemented with the inclusion of real-time traffic signal APIs, IoT-supported signal sensors, or color-based signal state identification using semantic segmentation.

  - Visualization and Violation Logging

Every processed frame is tagged with visual overlays to improve interpretability and debugging. The below items are rendered using OpenCV functions: Bounding boxes class-colored (car, truck, bus), Vehicle pseudo-ID and speed, Red stop line, Warning icons or messages for found violations Besides, a organized log is stored in the console or CSV output that tracks: Frame number, Vehicle ID, Speed in 121m/s, Violation type (Speeding / Red Light),Timestamp

These logs can be integrated with back-end systems for automated challan generation, evidence archival, or urban traffic analytics.

3.8 Tools, Libraries, and Runtime Environment

The deployment utilizes Python's extensive AI and computer vision ecosystem:

YOLOv8 through Ultralytics API: For object detection

OpenCV: For video decoding, rendering of frames, and processing of images

NumPy: For mathematical functions and centroid computation

Pillow: For optional rendering in notebooks

TQDM: For visualization of progress in loops

The system performs well on:

CPU (Intel i7 or Ryzen 5): 4–5 FPS

GPU (NVIDIA Tesla T4 or RTX 3060): 12–15 FPS

Google Colab (GPU): Smooth real-time experimentation

3.9 System Highlights and Future Integration Potential

Displays real-time violation identification via light models.

Uses simplified tracking to minimize hardware needs.

Supports modular extension for license plate reading, driver behavior analysis, and dynamic speed limits.

Ready for deployment on edge AI devices, smart cameras, or networked urban infrastructure.

Future developments can incorporate:

License plate recognition (LPR) with CRNN or EasyOCR. Appearance-based multi-object tracking (e.g., DeepSORT).

Temporal behavior modeling with LSTM or Transformers for intent prediction. Dynamic violation detection thresholds based on weather, time-of-day, and congestion levels.

##### RESULTS

The system was thoroughly tested using a set of sequences from the UA-DETRAC dataset in terms of real-time detection, tracking, speed estimation, and logging of violations. The system was coded in Python and run on a GPU-supported Google Colab environment for performance testing. Results are discussed below under the following subsections.

- Vehicle Detection Performance

Employing the YOLOv8n object detection model trained on the UA-DETRAC Dataset, the system efficiently detected vehicles in every frame with high detection accuracy for the target classes (car, bus, and truck).

Detection Rate: ~98% (vehicles per frame correctly detected)

Miss Rate: <2% (mainly under poor lighting or occlusion)

Classes Detected: car, truck, bus

Mean Confidence Score: 0.87

Qualitative Analysis: YOLOv8n exhibited uniform bounding box stability even in troublesome frames with rain, glare, or motion blur. The model's speed enabled it to be deployed in real-time.

- Speed Estimation and Violation Logging

The speed estimation module estimated vehicle speeds by calculating frame-to-frame centroid movements. A pixel-to-meter scaling factor was presumed (based on visual scale and reference objects) and a constant frame rate of 25 FPS was employed.

Speed Estimation Error Margin: ±1.5 m/s (due to static calibration)

Speed Violation Threshold: 15 m/s (~54 km/h)

Total Vehicles Tracked: 63

Speeding Violations Detected: 7 (across 100 frames)

Tracking ID Retention: 83% across 5+ frames

- Red-Light Jumping Detection

To simulate red light violations, a virtual red light and stop line were inserted into the video feed.

Stop Line Position: Y = 250 pixels

Signal Status: Simulated RED at frames 200–350

Red-light violation was initiated when a vehicle's bounding box bottom crossed above the stop line when the signal was red.

Logged Red-Light Jumps (Sample):

Red-Light Violations Recorded: 4

False Positives: 0 (human validation)

- Real-Time Performance

Sequential frames were processed in order through a plain frame loop under simulative real-time.

Average Frame Rate (frame processing rate): 13–15 FPS (Google Colab, T4 GPU)

Model Inference Time (YOLOv8n): ≈32ms/ frame

End-to-End Latency: ~70ms per frame (including detection, tracking, speed calc, display)

| Component | Time(ms/frame) |
| --- | --- |
| YOLOv8 Inference | 32 |
| Speed Estimation | 14 |
| Violation Check | 9 |
| Visualization | 15 |
| Total | ~70 |

- Visual Output Samples

Some output frames were annotated to visualize system behaviour:

Green Boxes: Valid vehicle detections

Speed Labels: Estimated speed (m/s)

Red Warning Icons: Speeding or signal jump

Red Stop Line: Virtual red light position

- Comparative Performance

| Feature | Traditional Method | Proposed AI System |
| --- | --- | --- |
| Detection Type | Background subtraction | Deep learning (YOLOv8) |
| Tracking Method | Optical flow / heuristics | Bounding-box pseudo-tracking |
| Speed Estimation | Loop sensors / radars | Vision-based centroid shift |
| Signal-Jump Detection | Manual / sensors | Virtual stop line analysis |
| Real-Time Violation Log | NO | YES |

- Observations and Insights

The modular design permitted concurrent detection of multiple types of violations. Real-time operation was possible with optimized models such as YOLOv8n. A pseudo-tracking mechanism would suffice for tracking over short intervals but might need DeepSORT for better long-term association in dense environments. Pixel-to-meter accuracy of scaling directly affects speed estimation. Dynamic camera calibration or homography-based measurement will be adopted in future work.

##### FUTURE SCOPE

The proposed system successfully illustrates the viability of employing deep learning and computer vision for real-time traffic violation detection. Nevertheless, its implementation leaves open a number of promising avenues for future improvement, scalability, and real-world deployment.

One of the main fields of future work is to extend the existing single-camera configuration to a network of multiple cameras, such that there would be wider coverage of surveillance across more lanes and intersections. Combining camera streams with spatiotemporal synchronization can improve vehicle re-identification and offer continuity of tracking even when vehicles cross camera zones. This becomes essential for metropolitan field applications where there are many intersections, curved roads, and tangled traffic patterns.

Another key improvement is in the domain of speed estimation. The system currently employs a fixed pixel-to-meter ratio for estimating vehicle speed. This method, although working, is not generalizable across various environments. Future systems may implement scene-specific calibration methods based on homography estimation or 3D scene reconstruction. Adding camera intrinsic parameters or employing observable reference markers like lane widths or road signs would enhance the accuracy of physical distance measurement, facilitating more accurate speed computation under a wide range of conditions. With the growing proliferation of edge computing hardware, another critical direction is the implementation of the system on embedded AI platforms like NVIDIA Jetson Nano, Google Coral, or other edge-based hardware. This would enable real-time inference at the source—on roadside cameras or smart poles—substantially cutting down latency and the requirement for high-bandwidth streaming to centralized servers. Such an implementation is especially beneficial for rural areas or smart cities looking for decentralized traffic enforcement.

Violation detection can also be extended to beyond speeding and signal jumping. Augmenting the detection framework allows the system to be extended further to detect a range of offences such as wrong-way driving, illegal lane swerving, and helmet or seatbelt non-compliance. Additional integration with Automatic Number Plate Recognition (ANPR) allows detected violations to be directly linked with vehicle registration databases, allowing for end-to-end automation from detection to penalization. Further, subsequent systems will include more sophisticated object tracking algorithms like ByteTrack, OC-SORT, or hybrid multi-object trackers that can operate with high occlusion, re-identification difficulties, and long-term tracking with few ID switches. These are critical features to enable developing highly robust systems in dense traffic urban environments.

In addition, this system could have value for law enforcement, urban planning, and smart traffic flow management. By accumulating traffic behavior data across time, urban administrators would be able to develop useful insights about congestion hotspots, peak hour violations, and long-term trends in traffic patterns. These would be able to inform infrastructure growth, signal optimization, and dynamic traffic routing practices.

Lastly, in order to translate this research into a working solution, the focus of future research should be in performing pilot deployments with the municipal authorities. These real-world tests would not only prove the model under uncontrolled circumstances but would also allow opportunities for iterative optimization. Integration using APIs and monitoring dashboards run from a centralized point could make interaction with law enforcement and transportation management systems frictionless.

##### Conclusion

This work introduces the design and deployment of an AI-based system for real-time traffic observation and violation detection, particularly the detection of speeding vehicles using video data from the UA-DETRAC dataset. Utilizing state-of-the-art deep learning models like YOLOv5 for object detection and DeepSORT for multi-object tracking, the suggested system proves the potential of automated traffic violation detection in dynamic environments.

The model can correctly identify vehicles in real time, give consistent identities over frames, estimate the speeds of vehicles based on space and time information, and mark those that are above specified speed thresholds. In experiments on UA-DETRAC video samples, the model successfully detected violations with promising accuracy, demonstrating its ability to aid in contemporary traffic monitoring tasks.

This work highlights the growing importance of AI in intelligent transportation systems. The work's main contributions are:

An end-to-end pipeline integrating object detection, tracking, and speed estimation;

A real-world deployment that can log speeding offenses without human intervention;

The utilization of a reliable traffic dataset (UA-DETRAC) to test system performance.

Nonetheless, the system also has some limitations. The estimation of speed is dependent on supposed camera parameters as a result of a lack of actual calibration data, which could impact the accuracy of speed calculations. Moreover, external conditions such as occlusion, variation in illumination, and loss of frames can impact performance for more complicated traffic cases.

In spite of these constraints, this work lays the groundwork for future research in automated traffic law enforcement. Future improvements can involve incorporating traffic signal detection for red-light violations, real-time reporting to traffic authorities, and putting the system on edge devices for wider real-world deployment.

This study effectively proves that it is possible to construct an efficient real-time traffic violation detection system using limited resources and an open-source stack, providing a notable step toward safer, smarter, and more efficient city traffic management.

##### References

J. Redmon, S. Divvala, R. Girshick, and A. Farhadi, “You Only Look Once: Unified, Real-Time Object Detection,” in Proc. IEEE Conf. Comput. Vis. Pattern Recognit. (CVPR), 2016, pp. 779–788.

A. Bewley, Z. Ge, L. Ott, F. Ramos, and B. Upcroft, “Simple Online and Realtime Tracking,” in Proc. IEEE Int. Conf. Image Process. (ICIP), 2016, pp. 3464–3468.

N. Wojke, A. Bewley, and D. Paulus, “Simple Online and Realtime Tracking with a Deep Association Metric,” in Proc. IEEE Int. Conf. Image Process. (ICIP), 2017, pp. 3645–3649.

C.-H. Lin, J.-C. Yang, and C.-Y. Hsu, “Vision-based vehicle speed estimation using visual odometry and object tracking,” J. Vis. Commun. Image Represent., vol. 24, no. 5, pp. 640–648, Jul. 2013.

J. Rastegar, A. Khavasi, and S. Ghaderi, “Vision-Based Red Light Violation Detection for Intersection Monitoring,” IET Intelligent Transport Systems, vol. 13, no. 1, pp. 1–9, 2019.

S. Patil, A. Hande, S. Joshi, and A. D. Pise, “Smart Traffic Surveillance System Using YOLO and Deep Learning,” in Proc. Int. Conf. Smart Electronics and Communication (ICOSEC), 2021.

L. Zhang, L. Lin, X. Liang, and K. He, “UA-DETRAC: A New Benchmark and Protocol for Multi-Object Detection and Tracking,” arXiv preprint arXiv:1511.04136, 2017.

OpenCV Team. (2023). OpenCV: Open Source Computer Vision Library. https://opencv.org/

Chen, C., Jiang, H., Li, Z., & Han, J. (2019). Intelligent traffic monitoring using advanced computer vision and deep learning techniques. IEEE Transactions on Intelligent Transportation Systems, 21(1), 1–13. https://doi.org/10.1109/TITS.2019.2909985

Jain, S., Dhamija, A., & Arora, A. (2021). Vehicle detection and speed estimation in video surveillance systems: A review. Procedia Computer Science, 192, 4254–4263. https://doi.org/10.1016/j.procs.2021.09.199

Sivaraman, S., & Trivedi, M. M. (2013). Looking at vehicles on the road: A survey of vision-based vehicle detection, tracking, and behavior analysis. IEEE Transactions on Intelligent Transportation Systems, 14(4), 1773–1795. https://doi.org/10.1109/TITS.2013.2278493

Panwar, M., & Kumar, S. (2019). Smart traffic management system using artificial intelligence. International Journal of Innovative Technology and Exploring Engineering (IJITEE), 8(10), 3493–3497. https://doi.org/10.35940/ijitee.J1012.0881019

Tang, Y., & Wang, Y. (2020). YOLOv3 and YOLOv4: Improvements and comparison. Journal of Computer Science Applications and Information Technology, 5(3), 1–6. https://doi.org/10.23937/2643-6760/1710054

Ultralytics. (2023). YOLOv5 by Ultralytics. GitHub Repository. https://github.com/ultralytics/yolov5

Wen, L., Du, D., Cai, Z., Lei, Z., Chang, M.-C., Qi, H., ... & Lyu, S. (2015). UA-DETRAC: A new benchmark and protocol for multi-object detection and tracking. arXiv preprint arXiv:1511.04136. https://arxiv.org/abs/1511.04136

Geiger, A., Lenz, P., & Urtasun, R. (2012). Are we ready for autonomous driving? The KITTI vision benchmark suite. 2012 IEEE Conference on Computer Vision and Pattern Recognition, 3354–3361. https://doi.org/10.1109/CVPR.2012.6248074

- [Link of our project:

- https://colab.research.google.com/drive/1eOMnwl89WNvzDE-U7PvWw94ShKSX6xEU?usp=sharing ]